# CTAM

# Table Of Contents

Contents

Table Of Contents	1

Introduction	2

Getting Started	2

1.	Cloning CTAM Repository	2

2.	Environment Setup	2

3.	Discover Test Cases	3

4.	System Discovery	3

5.	Basic Test Cases	4

Enable & Verify AC Cycle Test Case	4

Execute Service Validator	5

Full FW Update Flow	5

# Compliance Tool for Accelerator Management

# Introduction

The OCP Test & Validation Initiative is a collaboration between datacenter hyperscale’s having the goal of standardizing aspects of the hardware validation/diagnosis space, along with providing necessary tooling to enable both diagnostic developers and executors to leverage these interfaces.

Specifically, the ocp-diag-ctam tool provides acceptance testing for Accelerator Management in cloud data centers.

Note: -

- To get all compliance test cases to run, mandates a lot of collateral to be readied. This can get overwhelming. This guide should help you with getting used to the tool while incrementally enabling all the collateral.

- Please follow the following steps in the same order. At any point/step, it is assumed that the previous steps PASS indicating that the minimum collateral is met.

# Getting Started

## Cloning CTAM Repository

- Clone repo from GitHub to respective directory - git clone https://github.com/opencomputeproject/ocp-diag-ctam.git

- Change branch to developer.

- Create your own branch from developer branch If you want to work on this repository.

## Environment Setup

- Install python with version python-3.11.

- Open command prompt and run the below commands.

- Create a Virtual Environment for python using command python -m venv <virtual environment name>

- Activate virtual environment by running command <path to your virtual environment>/Scripts/Activate

- Install required packages from pip-requirements.txt file.

- For installing requirements run command python -m pip install -r <pip-requirements .txt>

## Discover Test Cases

- This acts as a test for the local repo setup. Optionally you can explore further and add more test cases along similar lines and rediscover the new set of test cases.

- Check for the ctam.py file is present in the current working directory. If not, then go to the directory or else give the path to ctam.py while running the command.

+----------------------------------------------------------------------------------------------------------------------+

|                                                  		Test Case info table				                        |

+---------  +-------     -----+----------+-------------+--------------+--------------+--------------------------------+

|  GroupID  |  GroupName  |  GroupTag  |  TestCaseID  | TestCaseName | TestCaseTag  | TestCaseWeightScore   |

+----------+-----------------+------------+---------------+------------------+---------------+---------------------+

|         	    |   		   |        []          |    TestID           | Test_01              |     []                   |          10                     |

| Group   |  	Group	   |        []          |    TestID           | Test_01              |     []                   |          10                     |                 |     ID    	    |   	Name	   |        []          |    TestID           | Test_01              |     []                   |          10                     |

|         	    |   		   |        []          |    TestID           | Test_01              |     []                   |          10                     |

+----------+-----------------+------------+---------------+------------------+---------------+---------------------+

Run the command to discover all the test cases python ctam.py -l

## System Discovery

- Ensure that the system is reachable and BMC & HMC are communicable.

- To run anything we need this properties in dut info, redfish config uri and .netrc file in our workspace directory

- dut_info.json

"properties": {

"ConnectionIPAddress": {

"description": "Default prefix for connecting to redfish",

"type": "string",

"value": "<>"

},

"DefaultPrefix": {

"description": "Default prefix for connecting to redfish",

"type": "string",

"value": "<>/redfish/v1"

}

}

- Redfish_config_uri.json

{

"GPU":{

"GPUMC": "<>",

"BaseURI":"<>/redfish/v1",

}

}

- .netrc file

<IP ADDRESS 1>

<USERNAME 1>

<PASSWORD 1>

- Make sure all the config files should have inside workspace directory and we need to provide the workspace directory while running the command as command line argument like below.

- Using this configuration, we can run python ctam.py -w <workspace_directory> -d

<add sample o/p>

## Basic Test Cases

- The simple checks will help build the confidence to run FW update test case later.

- Test Cases

- Python ctam.py -w <workspace_dir> -t <test_case_id or test_case_name >

- Simple/Basic/Fundamental Test Cases

  - H10 - CTAMTestRedfishTaskService

  - H7 - CTAMTestRedfishTelemetryService

  - H4 - CTAMTestRedfishUpdateService

  - H8 - CTAMTestRedfishEventService

- FW/SW Inventory

  - H5 - CTAMTestRedfishFirmwareInventoryCollection

  - H6 - CTAMTestRedfishFirmwareInventoryExpandedCollection

  - H9 - CTAMTestRedfishSoftwareInventoryCollection

  - H11 - CTAMTestRedfishSoftwareInventoryExpandedCollection

- Other Test Cases (optional, any failure for the test cases below should not prevent you from moving ahead with the document)

  - H96 - CTAMTestLogserviceDumpClearlog

  - T2 - CTAMTestTelemetryMRListRead

  - T4 - CTAMTestTelemetryMRRead

  - H97 - CTAMTestLogServiceDumpURIListRead

  - H99 - CTAMTestLogServicesURIListRead

  - H83 - CTAMTestRedfishEventServiceCreateSubscription

  - H81 - CTAMTestRedfishEventServiceDeleteSubscription

  - H80 – CTAMTestRedfishEventServiceSubscription

- <add sample o/p>

# Enable & Verify AC Cycle Test Case

- This will be needed for FW update later.

- To run the below test case we need this properties in dut info, redfish config uri and .netrc file with default configuration.

- Make sure we have the required default configuration files in the workspace directory.

- python ctam.py -w <workspace_dir> -t <test_case_id or test_case_name >

- AC Cycle

  - H100 - CTAMTestAcCyclesInLoop

  - python ctam.py -w workspace -t   H100

# Execute Service Validator

- This is needed for confidence (any failure for the below test cases should not prevent you from moving ahead with the document)

- Make sure we have the required default configuration files in the workspace directory.

- python ctam.py -w <workspace_dir> -t <testcase or test_case_name >

- Redfish Service Validator

  - T0 - CTAMTestServiceValidator

  - python ctam.py -w workspace -t   T0

Note: -  We need to mention in our test case which URI we are going to validate. For further information please refer to this GitHub link - Redfish Service Validator

# Full FW Update Flow

- For running firmware update test cases we need additional configuration file which is package_info.json in the workspace directory.

- In package_info.json file we need to add the configs below.

- {

"GPU_FW_IMAGE": {

"Path": "directopry from where we need to take the file.",

"Package": "<fwpkg_file_to_update.fwpkg>",

"Version": "version",

"JSON": "<fwpkg_json_file.json>",

"Vendor": "Vendor Name"

}

}

- First, we need to do precheck to check if the device is updated or not.

- If updateable then we need to stage, the firmware (copy the firmware file into target device).

- Then with a power cycle we are activating the firmware.

- After firmware update completed we need to verify that if the firmware is updated correctly or not. If verified, then test case passed else fail.

- python ctam.py -w <workspace_dir> -t <testcase_id or test_case_name>

- Full FW Update

  - F1 - CTAMTestFullDeviceUpdate

  - python ctam.py -w workspace -t   F1

# Full FW Rollback Flow

- For running firmware update test cases we need additional configuration file which is package_info.json in the workspace directory.

- In package_info.json file we need to add the configs below.

- {

"GPU_FW_IMAGE_BACKUP":{

"Path": "directopry from where we need to take the file.",

"Package": "<fwpkg_file_to_update.fwpkg>",

"Version": "version",

"JSON": "<fwpkg_json_file.json>",

"Vendor": "Vendor Name"

}

}

- First, we need to do precheck to check if the device is updated or not.

- If updateable then we need to stage, the firmware (copy the firmware file into target device).

- Then with a power cycle we are activating the firmware.

- After the firmware update is completed, we need to verify that if the firmware is updated correctly or not. If verified, then test case passed else fail.

- python ctam.py -w <workspace_dir> -t <testcase_id or test_case_name>

- Full FW Update RollBack

  - F1 - CTAMTestFullDeviceUpdateRollback

  - python ctam.py -w workspace -t   F0